Memorandum

To:		Francisco Contreras

From:	Bob Cheung – Transportation Division

Date:	August 8, 2012

Re:	8497-8499 Sunset Boulevard

This memorandum summarizes the traffic impact findings for the subject project.  The revised project proposes to provide: 4,528 sf of retail, 11 units of residential condo and 6,712 sf of quality restaurant uses. A traffic impact study for a recently-approved, larger project proposed by the same applicant at the same site was previously analyzed as part of an EIR which proposed/analyzed: 3,007 sf of retail, 34 units of residential condo and 6,153 sf of high-turnover restaurant.  The EIR for this previous proposal was certified by the City Council on January 18, 2011.

Based on the changes of land uses and their respective densities, a trip generation analysis was prepared to assess the likelihood of potential new traffic impacts that were not identified in the aforementioned EIR.  Based on the proposed project, the trip generation estimates are summarized in the table below:

As compared to the project trips that were analyzed for the previous project’s EIR, the revised project would generate less trips as follows:

The findings in the certified EIR concluded that the project would not create any significant traffic impacts at any of the study locations with the exception of the intersection of Sunset Boulevard/La Cienega Boulevard/Miller Drive.  The EIR also concluded that the significant impact would be fully mitigated with intersection improvements.  Given the revised project would generate less trips, it can be concluded that the revised project would not create any new significant traffic impacts beyond which were identified in the EIR.  Also, since the only significant traffic impact identified in the EIR at Sunset Boulevard/La Cienega Boulevard/Miller Drive was fully mitigated, it can also be concluded that the mitigation measure would also fully mitigate the revised project impacts.  The mitigation measure that was required in the EIR has been included as a mitigation measure for this project.